An Architects Perspective on Bidding: How to ensure the lowest possible construction costs. 
Submitted by:  Miles Yanick, Architect, LEED AP

Allocation of Risk
Open Bid:  
Highest risk to the contractor.   Highest cost to the Owner.
Negotiated Bid: 
Medium risk to the Owner and Contractor.  Average cost to the Owner.
Cost of Work: 
Lowest risk to the Contractor;  highest risk to the Owner.  Lowest cost to the Owner.

During the design process I am often asked how to insure the lowest construction cost of a project.  








Whether the 
Owner has set a maximum construction cost budget or not, it is necessary during the preliminary design stages of the project to develop a construction cost estimate.  We provide construction cost estimates based on local average square foot or cubic foot costs for comparable projects.  We also use a somewhat more detailed preliminary cost estimating system that we have developed in-house over the years.  Neither of these systems provides a completely detailed construction cost but they do serve as a guide during the remainder of the design and specification process.
























































































Once a general idea of the cost is identified the Owner needs to know how we intend to meet this preliminary budget and how we will keep the costs within this figure.  In too many cases the Owner believes the best method to obtain the lowest cost for a construction project is to send the drawings and specifications out to bid (usually three bids).  My response to this is that if we want to insure the lowest cost, the last thing we want to do is send out for bids.  Bidding doesn’t necessarily insure the lowest construction cost.  I virtually always recommend a negotiated contract of some sort with a general contractor.  This contract can be either a Negotiated Bid or a Cost of Work type contract. 
The bidding process open to all potential bidders often sets the stage for the entire project.  If the general contractor wants the opportunity to be awarded the project they often shop bids from the cheapest sub contractors.  Since such a large part of the project is accomplished by these sub contractors, which one of the cheapest subcontracts will the general contractor use as part of his/her bid?  The Owner may get the lowest bid but not necessarily get the best or fairest price for the work.  
The lowest cost for the work still may not be reached even if a limited group of three or four general contractors are asked for bids.  Each contractor has to factor in the cost of all the “what ifs” that happen on the job.  Things like: what if there is a weather delay; what if a sub contractor provides the wrong material, etc.  Each contractor will factor these “what ifs” differently and the cost of each will be included in the bid amount.  Of course if none or only some of these “what if’s” occur, the Owner will pay for them anyway in the bid amount.  There is also the actual cost of preparing the bid.  (There is no such thing as free estimates or bids).  A small remodel project may only cost the general contractor a thousand dollars or so to prepare the bid, while a larger project could cost many thousands.  This cost will be reflected to the Owner in the bid amount.  If the lowest bid is selected then that number becomes written in stone.  Any changes or revisions in the work have to be accomplished through Change Orders which can add significant negotiation and administration cost to the project in additional fees charged by the architect, engineers, etc.

The bidding process has additional perils as it takes considerable time.  This could be as little as a month for a small remodel project, to several months for a larger one.  There are also the additional professional architects and engineers fees needed to prepare a thorough and complete set of bidding drawings and specifications and the reproduction costs for these documents.
As an alternative to the bidding process, I recommend to Owners that we avoid the complexities and perils of the bid process altogether.  I recommend that the Owner and I select several reputable contractors for consideration as potential general contractors as soon as we have design development preliminary drawings completed.  These contractors can be suggested by the Owner or my office.  We then interview each of the contractors who are asked to present their firm qualifications, past work references, cost control methods, how they might respond to design alternatives, etc.   They also can field other questions from the Owner and myself.  Together with the Owner, we make a selection of a contractor we prefer to work with through the completion of the design.  This way we get early and reliable cost alternatives, design input and constructability reviews from a general contractor’s point of view.  It is made clear to this contractor that these services are for construction cost estimating and design input purposes only and do not constitute his/her selection as general contractor for the construction of the project.  The contractor is paid for these services.  The final cost figures prepared by the contractor become the property of the Owner as instruments of service.  At the end of the design process we have a firm project construction cost and the Owner has had the opportunity to work with the contractor.

Now the determination has to be made whether to bid or not to bid for the construction of the project.  At this stage of the project, Owners often want to send out for bids just to see if any bids come in lower than the estimated cost.  All of the aforementioned problems are still in play if the project goes out to bid.

If a relationship of trust has been developed, the Owner can select the pre-construction contractor to be the General Contractor for the construction of the project.  Other construction contract options are also available to the Owner once a solid construction cost has been established.  One method of obtaining the lowest cost is the Negotiated Bid.  This process is to select the General Contractor to work with and have him/her prepare a Lump Sum Bid for the work.  The advantage to the contractor in this situation is that he/she doesn’t have to bid against any other contractors.  There is no need to select the lowest sub bids just to get the job.  The entire bid package including all the sub bids is then reviewed with the Owner and Architect.  If either feel a certain sub bid price or sub contractor is not right for the work, then these cost items or sub contractor(s) in question are negotiated until the total bid amount is arrive at and a contract is signed.

Another method of obtaining the lowest cost is the “Cost of Work” type contract.  This contract methodology has the lowest risk to the contractor; and it usually produces the lowest cost to the Owner.  The same qualification and interview methods are used to select the general contractor as with the Negotiated Bid type contract.   When the contractor is selected then he/she prepares a detailed estimated cost breakdown for the entire project.  This breakdown includes every possible contingency and assigns an estimated cost to each.  If that contingency doesn’t occur, then there is no cost to the Owner.  The Owner pays only for the actual cost to the contractor of all materials and labor plus the contractor’s normal mark up, not for the estimated costs of any “what ifs” that have to be included in an open bid type contract.  That way the Owner only pays for material and labor actually performed on behalf of the project.  The overall cost risk here is taken by the Owner but it pays off almost all the time.  And even if it doesn’t pay off in the lowest cost, the Owner knows exactly why.  
I recommend this type of relationship with general contractors on virtually all new home and remodel projects.  It takes less time than the open bid process and our professional fees can be somewhat lower if we are allowed to work with a contractor we’ve had successful projects with in the past.  This also means our construction drawings and specifications can be abbreviated somewhat, resulting in lower professional architectural and engineering fees and lower construction contract administration fees.  Last, but not least, when the project is finished, the Owner knows exactly what he or she paid for. 
Contact:  Miles Yanick, LEED AP

Phone:     206-842-6516

Email:
    milesyanick@milesyanick.com

An Architects perspective on the “Let’s get three bids”

Page 1 of 3

